ACE Pilot Model Simulation Program

D.W.Davies  August 1999

Introduction

The ACE Pilot Model began operation in May 1950 and continued to be developed until it went into operation as a service. The version simulated is the one described by James Wilkinson in Trans. Roy. Soc in 1955 (1).

Martin Campbell-Kelly also wrote about programming this machine in 1981 (2). This note assumes a general knowledge of the machine and its programming which can be obtained from these two papers.

A very brief description of the machine is followed by an outline of the way in which the simulation program handles its memory and its main functions. There follows a description of the controls of the program and how to use them, with some examples of programs that can be run. Finally the deficiencies of the simulation are described. A list of sources and destinations is at the end.

The simulation is programmed in Visual Basic, Version 1, which is no longer available as a product, but the program can be copied and distributed together with enough software to run it in a Windows environment. It should be easy to adapt it for a current Visual Basic system.

The ACE Pilot Model

The use of mercury delay lines for the main working memory creates a problem of access time. Alan Turing made the first logic design for ACE, of which this was a pilot model, with clever features to minimise waiting time, as long as the programmer used the facilities well. This technique became known as optimum programming. However it was an additional task for the programmer, so this was a machine for the dedicated and professional user, and not easy to get familiar with.

As a consequence, the ACE Pilot Model was certainly the fastest machine when it was new. The advent of true random access memories, in the form of core stores, made optimum programming unnecessary, so the Pilot Model and ACE which followed it became of little significance in the development of computing technology, except as an interesting side issue.

Because if its speed and the mathematical ability of the team which exploited it, as well as the organisation which ran it, the Pilot Model did excellent and valuable work, which is detailed in reference 2. It led to the Deuce, an engineered commercial version made by English Electric and a curious magnetic Drum version by Bendix in the US of which 400 were sold.

Outline of the Pilot Model

The main working memory was 11 ‘long’ delay lines each storing 32 words of 32 bits. In addition there were 5 ‘short tanks’ holding one 32 bit word each, for rapid access and 2 delay lines holding 2 words each, suitable for holding 64 bit quantities but able to hold two 32 bit words if you wished. The clock rate of 1 Mbit/s made a word time of 32 microseconds, called a minor cycle and the whole long delay line cycle time of 1024 microseconds called a major cycle.

Instruction were 32 bits long, stored in long delay lines (DLs). They were placed in the DLs in such a way that, with good programming, an instruction was available as soon as it was needed. This, with the judicious use of the one word stores (TS) or two word stores (DS) made the machine fast. At its best a short operation occupied two minor cycles, one for instruction set-up and one for the operation.

To help this optimisation process, each instruction had two timing numbers and a characteristic of 2 bits which controlled when the operation started and finished and when the next instruction would be taken (its location in the DL). This flexibility enabled operations lasting anything up to 32 minor cycles, which were well exploited in expert programming. It also gave the opportunity to locate instructions in any gaps remaining in the DL, using memory better at some cost in time. The most frequent loop in a program (which determined its speed) must be a multiple of a major cycle in length and often was just one or two major cycles, i.e. 1-2 milliseconds approximately. Very short loops could be ½ major cycles long if the instructions were written twice, but this was a rare refinement for important subroutines. Two examples are among the programs available for the simulation (see below).

The other special feature of the Pilot Model was that the instruction had no ‘function’ field. It could only send words from one source to one destination. As well as the sources and destinations for the memories, there were others which had special functions. For example the double word memory DS14 was accessed at source 14 and destination 14, while source 13 gave the contents of DS14 divided by two and destination 13 added the word from the source into DS14, making it a double word sized accumulator. TS16 was a single word accumulator with add and subtract inputs, at destinations 17 and 18 respectively. Logical operations were performed on the contents of TS26 and TS27 using sources 17, 21 and 22 respectively. These source and destination numbers seem arbitrary and derived from the history of design and redesign in the period when the Pilot Model was being studied by trial programming. At the end of this note the list of Sources and Destinations is given, with a summary of the way the timing of operations behaved.

Timing of the operations depended on two 5 bit fields in the instruction, Wait (W) and Timing (T). The 2 bit characteristic (C) determined 3 types of operation, since value 2 was not used.

The C value 1 gave a single word transfer. If the instruction was in minor cycle M, the transfer took place at minor cycle M+W+2, then the next instruction was taken from minor cycle M+T+2, but since it must happen after the transfer, it could be M+T+34 in real time. M+T+2, modulo 32, gives the place of the instruction in the DL.

The C value 0 gave a transfer of a number of words. This began at M+W+2 and continued until M+T+2, when the next instruction was taken, a total of T-W+1 minor cycles, or T-W+33 if T<W. Thus W=5 and T=4 gives a 32 word operation starting in M+7 and ending in M+38.

The C value 3 gave a 2 word transfer starting in M+W+2, like all the others, with the instruction taken from the word at minor cycle M+T+2, modulo 32 in the DL, at the first opportunity after the transfer finishes.

These rules are summarised in the table at the end of this note. In practice very little arithmetic is needed to place instructions correctly, but there is a chance of troublesome errors at this stage of writing a program.

The two references give a good account of the functional sources and destinations. In the simulation there were subtle details associated with adding into DS14 and with the use of TS20 as a delayed access to DL10 when a trigger TCA is set. These are treated in the documentation of the programs.

Outline of the simulation

A problem arose from a design feature of the version of Visual Basic (for which there is no possible justification). Variables declared as long have 32 bits but if ever bit 31, the sign bit, becomes 1, an overflow is signalled and the program comes to a stop from which it cannot be rescued! Thus even simple logical operations on 32 bit words are impossible. The same happens to 16 bit words declared as integer if bit 15 becomes 1.

Since  the Pilot Model did not have this stupidity, it became necessary to represent every 32 bit word in two long variables, using (most of the time) only the lower 16 bits. This applies to all words, even those stored in a file which represents the drum

Thus the main memory is in two arrays MemH and MemL for the top and bottom halves of the 32 words. Delay lines 1 to 11 are in the entries MemH(32) to MemH(383) and MemL(32) to MemL(383). Entries MemH/L(0) represent the instruction register and MemH/L(1) the input register, known as the input dynamiciser which responded either to a binary number set up on lamps on the control panel or to words being read from cards. MemH/L(28) represent the output register, known as the output staticiser, which lit 32 lamps on the control panel or punched words on cards.

The contents of TS and DS memories have there own place in the MemH and MemL arrays, which are shown, together with the above details, in the table of sources and destinations.

The main subroutines which perform the simulation are Transfer and Oneshoot.

Transfer does the operation of the Highway for just one minor cycle, transferring a word from a source to a destination. It performs the logical and arithmetic operations associated with sources and destinations, such as the accumulators TS16 and DS14. The multiplier is handled by a separate subroutine Mult and Drum operations by subroutine Drum.

Oneshoot performs the sequence of transfers invoked by a single instruction. It first parses the instruction to find the fields N, S, D, C, W, T and G. Then it interprets the values of C, W and T to perform one or more Transfer operations. The minor cycle of each transfer is held in ST but this is derived from TT which records the total number of minor cycles since the Pilot Model began its operation. After the transfers, Oneshoot loads the next instruction, taking into account the effect of a discrimination on Negative or Non-Zero (Destinations 24 and 25) by the value of Disc.

In Acepm3, card operations are part of Oneshoot. In Acepm2 they are in Runit, which is not so satisfactory.

To run the simulation, Oneshoot is repeated as long as the Go digit is 1, under the subroutine Runit, entered via Run-Click. Oneshoot can also be invoked by the One-Shot button when the Pilot Model is stopped.

Display is a complex routine to show the chosen two DLs and other registers. Its main subroutine to display a word is Print Word. Depending on the option button this invokes Inststring, Hexstring or Decstring to get the string for output in instruction, hexadecimal or decimal format, except for the instruction register which always shows instruction format.

Decstring is a more elaborate routine than the others and uses subroutines TimesTen, PlusK and MinusK.

Drum is invoked by destination 22 and its least significant C bit decides on read or write. The source value determines either a head shift or an actual read/write. Drum determines the track number (0 to 127) by combining Head and Shift numbers. It sets the DL to be used as DL9, but DLNo can be something else in manual drum operations. Having set these numbers, either DrumWrite or Drum Read does the work.

The drum is represented by a single Windows file C:\Vb\Acedrum, opened by Sub Form_Load as #1. This is a binary file with each of the 128 tracks as 64*4 bytes. These are MemH and MemL for each of the 32 words, both as 4 byte records, read into or written from the 4 byte long variables.

Cards, which may be read or punched, are represented by Windows files C:\Vb\Acecards and C:\Vb\Acepunch, opened by Form_Load as #2 and #3 respectively. These are organised by the same software as the drum and have tracks numbered from 128 to 255 and from 256 to 383 respectively. It is therefore possible to perform manual transfers between cards and between cards and drum tracks in manual operations, which is very useful for testing. In card reading operations by the Pilot Model’s programs, any card can be read. In card punching, normally only cards 256 to 383 are accessible, these corresponding to the stack of blank cards in the punch. This has been done to avoid accidental writing over cards kept for reading, such as program cards. Thus cards 128 to 255 can be regarded as ‘read only’ cards while 256 to 383 are ‘read-write’ cards. But it can be useful to write on the former set when preparing program cards, so a special (artificial) feature was added to the simulation that the use of source 30 (instruction 31- 30) will stimulate card punching on any card in the range 128 to 383. Initial input can take place from any triples of cards. Note that punching from a program is only on the first 12 words of a card image. To make satisafactory triples for initial input the 13th row (normally hex FFFF FFFF) must be added in a DL using drum read and write operations.

The subroutines ReadCard and PunchCard set the seek number of the binary file to the correct track or card number (called ‘Track’), set the card row number to 0 and set either CardReading or CardPunching to 1. Since reading and punching take place during running, the subroutine Runit does the operations on each row, numbered from 0 to 11. When it reaches row = 12, it stops the card operation and advances the card (track) number for the next card. Special features are needed for initial input, when Initial = 1, which are described below.

This arrangement has the snag that one-shot operations do not do the card reading or punching. This is, in fact, true of the real machine. But it could have been avoided, to aid testing, by placing the card operations in Oneshoot or Transfer.

It was impossible to provide a realistic T.I.L. source, which signifies the 12th row of a card, because real time is not simulated. An approximation is made, but any testing of T.I.L must be done after the stop instruction which reads or punches a card row.

Most of the other routines concern the many manual controls and a few test set-ups which facilitated debugging. Any words to be placed manually in DLs, TS, DS or other registers are placed in the top text box. They are converted as instructions, hexadecimal or decimal numbers according to the option chosen by ConvertI, ConvertH or ConvertD, the result being in Converted H and Converted L for the two halves.

The manual controls and displays on the form

The Visual Basic program uses one form, which is the ‘control panel’ for the operation of the simulated machine. It has some relation to the actual machine but differs in so many ways that a detailed description is needed to make it useful.

When the form is first loaded it is in skeleton form and lacks any data. Clicking a button such as ‘Read’ near the bottom right will do a drum read and bring up the contents of DLs and other memory to show what can be displayed.

N.B. The drum transfer caused by button ‘Write’ can destroy drum or card data and should be used with great care.

For our description we shall divide the panel into four columns. The left two show the contents of two delay lines, the third column shows the contents of TS, DS and other registers while the right hand column contains the various controls. These controls will be identified by their rows, of which there are ten.

The left two columns - Delay line displays

As in the actual machine, two chosen delay lines can be displayed, and these are identified as A and B. By clicking the buttons A and B at the top, the number of the delay line in question is incremented, so any one can be displayed at either column. When the form appears, DLs 1 and 2 are shown.

The 32 words are listed with the index 0 to 31 between the displays. The format of the words can be Hexadecimal (8 digits), Decimal (10 digits) or Instruction with the separate fields N, S, D, C, W, T and G of sizes 1, 2, 2, 1, 2, 2 and 1 digit respectively. This format is selected by the options buttons in row 3 of the controls column.

The main purpose of these displays is to read the instructions in a program and check progress while entering a new program. Instead of using punched card input, programs are entered into delay lines using the controls, as described below, then stored on drum tracks, as will be described. By clicking the ‘Read’ button after the form is first loaded, drum track 1 will have been loaded into DL1, shown in the first column as selection A. The format will be Hex and can be changed to Instruction in the ‘Word Format’ options group, so that the program can be read. This is the ‘Successive Digits’ program.

Unfortunately, display is so slow that to show a changing pattern in the DLs or other memory would make everything very slow indeed. Therefore nothing changes while a program is running (unless cards are being processed). To make it possible to see progress, the output staticiser register can be changed by a program, using destination 28, and the output appears in the text box labelled ‘Output’ on row 6.

The third column - TS, DS and other registers

Reading down the page, this column shows the contents of the instruction register, the input dynamiciser, the TS and DS memories and the output staticiser. For each DS, two words are shown with the even minor cycle or least significant at the top and the odd or most significant at the bottom.

Like the DL display, these displays only show changes when the machine stops running, for example at an instruction with G = 0. The output register however is duplicated in the output text box and this text box changes whenever something is sent to destination 28.

The format of all these registers is determined by the ‘Word Format’ option group, except that the instruction register is always shown in instruction format.

All these registers can be loaded using the controls in a way to be described below.

The fourth column - controls

These controls are arranged in ten rows, which will be described in turn

Rows 1 and 2

The top two rows enable words to be entered into any memory or register except the output staticiser. The word to be entered should be written into the top text box. If the word format is set to decimal or instruction, only numerals or spaces will be accepted, other characters ignored. If it is set to hexadecimal, letters A to F are also accepted and are written as capitals. Spaces are optional and are ignored when the word is used - their purpose is to make the format clearer, especially when instruction format is used. When the word format option is changed, the notation in the text box does not change, so characters such as letters may no longer be acceptable when the word is used for input.

The word in the text box can be placed anywhere in the DLs displayed as A and B. This is done with the A and B buttons in rows 1 and 2. The minor cycle into which they will be placed is determined by the small text box in the fourth row, which initially holds zero.

To select a minor cycle, this entry in the small text box can be written into or it can be changed by using the ‘Up’ or ‘Down’ buttons or returned to zero with the ‘Zero’ button. In this context, ‘Down’ means down the display, in other words incrementing the minor cycle number while ‘Up’ means reducing the minor cycle number.

For example, to load an instruction into word 5 of DL1, first ensure that DL1 is being displayed, for example as the left column or A display. Ensure that format ‘Instr.’ is the option chosen, then enter the instruction in the top text box, optionally using spaces to mark out the various fields. Having checked that the instruction is correct, set the minor cycle number in the small text box on the fourth row to 5, by writing in or using the ‘Down’ button if it is zero. The click the A button at the top of this column to enter the word into the DL. When either the A or B entry buttons here are used, the minor cycle increases by 1, modulo 32, ready for the next entry.

If the option says ‘Instr’ and the word in the box cannot be an instruction, for example it contains a letter or the source field exceeds 31, there will be a message box to warn the user and no entry will be made. For the decimal option, letters or a number larger than 232 - 1 will be rejected in this way. Some rejections may be the result of changing the format option between entering the word and trying to load it.

Row 2

In the second row are buttons ‘TS’, ‘Input’ and ‘Instr.’ for loading these registers. The TS button loads a TS or DS or the output register, according to the number in the small text box in the fourth row, which must be a TS or DS destination or the output destination 28. In the case of a DS, the word is entered into the even minor cycle and the number in the even minor cycle is moved to the odd. Thus a two-word entry is entered most significant part first.

The usual format restrictions apply to loading TS or DS or the output register in this way. The button ‘Input’ loads the input dynamiciser, as if the word in question was placed on the input lamps of the Pilot Model’s desk. The ‘Instr.’ button  accepts only valid instructions and places them in instruction format in the instruction register.

Row 3

This row has the ‘Word Format’ options buttons. These determine the format in which the contents of the top text box will be converted for loading into a DL, TS, DS or other register. They also change the notation of the display of the contents of the two chosen DLs and other memory locations, except for the instruction register, which is always in instruction format.

Row 4

Row 4 contains the small text box which determines which minor cycle a word will be placed in or which of the TS or DL destinations. The entry in this box can be manipulated by the ‘Up’, ‘Down’ and ‘Zero’ buttons or simply written in.

Row 5

The fifth row has text boxes ‘TCA’, ‘TCB’, ‘m.c.’ and ‘total m.c.’ The first two show the state of these triggers, TCA and TCB as ‘On’ or ‘Off’. They can be altered only by the program.

The ‘m.c.’ text box starts at 0 with a new form. It shows the minor cycle from which the current instruction has been obtained.

The ‘total m.c.’ text box shows how many minor cycles have elapsed since the program began. Only ordinary transfers are counted, making no allowance for  mult instructions or drum operations or waiting at a stop instruction during card processing.

None of the indications on the fifth row changes during a run, they show true values only when the run stops, for example at an instruction with G = 0.

Row 6

The sixth row is the output staticiser, which does show changes during a run.

Row 7

The seventh row concerns the drum and is used to make manual operations on drum tracks or card images. The tracks number shown in the text box refers to both tracks on the drum and card numbers and therefore runs from 1 to 383. The DL number runs  from 1 to 11. Both these numbers on this row can be written in to make a transfer between a track and a DL, using either of the ‘Read’ or ‘Write’ buttons. I have made a practice of writing programs in 5 consecutive tracks to avoid loss by ‘finger trouble’. The Drum is a file ‘acedrum’ in the Vb directory and is preserved when the simulation ends. Transfers between cards and DLs are artificial but are useful for preparing cards for reading.

The result of clicking ‘Read’ while using the initial values Track = 0 and DL = 1 is to read track 0 into DL1. It will consequently load the ‘successive digits’ program from track 0 into DL1, which initially is in display position A.

Row 8

The eighth row contains the main running controls. The Enter button loads the instruction register with the entry instruction of a program. For this purpose the DL of the entry point must be shown in the A position and the minor cycle number of the entry point in the text box on this line.

When ‘Enter’ is clicked, the instruction selected is placed in the instruction register and the m.c. text box on the fifth row takes the value chosen for the entry minor cycle, since that is the minor cycle of the current instruction. The ‘total m.c.’ number is cleared when ‘Enter’ is clicked or at initial input.

Having loaded the initial instruction, it remains only to click ‘Run’ to start the run, but make sure everything is ready, such as a number in the input register or numbers loaded into certain registers, otherwise the program may misbehave, typically by entering a infinite loop. The ending of a run is shown by a rewritten display. If the simulation is still running, all buttons are ineffective. This can best be tested using the Up or Down buttons.

It was intended that a running program could be stopped by means of a ‘Halt’ button but an oddity in the way Visual Basic works makes this impossible.

If a program is running and appears unlikely to stop, the only way to stop it is to press Control plus Break on the keyboard, then the ‘Run’ menu and ‘End’. If the simulation is using the .exe version of the software, it cannot be stopped in this way and Control+Alt+Delete is needed. This fortunately does not restart the computer but allows the program to be ended neatly.

Whenever the simulation is stopped, the ‘One-Shot’ button will advance it one instruction, which can help in debugging.

Row 9 and 10

The ‘Init. Cards’ button performs initial input, in which triples of cards are read to load delay lines. The number of the first card of the first triple must be placed in the ‘Read Card’ text box. The operation of this function is described below.

The ‘Read Card’ text box gives the card number of the first card in the simulated card reader, while the ‘Punch Card’ text box specifies the first card that will be punched. Their initial values are 128 and 256 respectively, the first cards available in these categories. Reading is possible from cards 128 to 383. Punching can only be done on cards from 256 to 383, but exceptionally if the punching is started by a 31 - 30 instruction the cards from 128 upwards can be punched. Programs which do this must first be well tested in case they wipe out programs or data stored on cards.

The Exit button stops the Visual Basic program running, but is ineffective while anything is happening, so the simulation program must have stopped. If it cannot stop, the Control+Break must be used, or Control+Alt+Delete, as mentioned above. See the next section for the use of the ‘Init. Cards’ button.

Initial Input

The Pilot Model employed a clever ‘bootstrap’ method by which a set of three punched cards could hold a DL full of program and load it into a delay line using only three instructions at the start of the first card to perform the loading process. The button ‘Init. Cards. On the ninth row starts this process with the card in the ‘Read Card’ text box as its first card of the triple.

This button (a key in the real machine) starts the reader, which must continue running until it runs out of cards. This is done by including a 13th word in the card image of some initial input cards. The Runit routine (Oneshoot in version Acepm3) reads this when it reaches the 12th word and if it is non-zero reads the next card. I have arranged that cards for initial input have all ones (Hex FFFFFFFF) on the 13th row except for the last card.

A separate document (ACE Initial Instructions) describes how the process is performed.

Examples of programs stored on the drum

All the programs have been stored in 5 tracks, to guard against loss by finger trouble. They all start in DL1, minor cycle 0, so zeroise the m.c. counter which is used to place words in DLs etc. before pressing ‘Enter A’ then ‘Run’.

In the following descriptions the numbers after the title give the track numbers of the drum track(s) containing the program (repeated 5 times for safety) and the numbers of the cards holding the initial input. To use the latter, enter the first card number in the ‘Read Card’ text box and click ‘Init. Cards’.

1) Successive Digits [in tracks 0-4, cards 128-130]

This was the first program which ran on the Pilot Model. It adds the input register to TS16 until that overflows and becomes negative. Each time that happens, TS16 is cleared and a bit in TS16 (which starts as P1) is shifted left and displayed in the output.

To operate this program:

Start Visual Basic with P5

Press ‘Read’ (i.e. load Drum track 0 to DL1, which is displayed in Hex.)

To see this as instructions press option Instr. of Word Format, but return to Hex

Put a suitable input constant into the top text box, such as 001 in hexadecimal

Press Input to load this into the Input register, which the program uses

Press Enter A, then Run

The output will show the bit progressing up the register. Decimal or hex output is interesting for this purpose. The rate increases for an increased number in the input register. With hex 001, each step takes about ¼ second and the run from right to left takes about 8 seconds. Then the run stops, allowing changes, such as the word format option or the input word value, before pressing Run again.

The 8 second run is timed at 2099168 minor cycles or 67.17 seconds in the pilot model. It is faster in the simulation mainly because the loop has only two instructions, both with a single transfer.

2) Squares and cubes of numbers up to 1625 [in tracks 5-9, cards 131-133]

For numbers from 1 to 1625 it forms the squares and cubes. At the end, only the square and cube of 1625 is retained. By one-shots, 16253, 16252 and 1625 are output, leaving the program ready to run again. This occupies 104005 minor cycles (64*1625+5) or 3.33 seconds. The simulation takes about 2 seconds.

When loaded by initial input, this program performs at once, stopping when ready for output.

3) Fourth powers, cubes and squares of numbers up to 255 [in tracks 10-14, cards 134-136]

Similar to the previous program. It occupies 16319 minor cycles or 0.522 seconds of Pilot Model time. Too short to measure simulation time. Performs after initial input.

4) Division program by non-restoring method [in tracks 15-19, cards 137-139]

Given a double length number in DS14 and a single length divisor in TS26, this program produces a rounded off quotient in TS16. It takes 1106 minor cycles or 35.4 milliseconds on the Pilot model.

To demonstrate the program, first use the multiplication test by loading TS15 and TS16 with numbers and pressing 15*16. Then place one of these numbers in TS26. Set the m.c. counter on the 4th row back to zero (Zero button) before pressing Enter A then Run to start the division. TS16 should contain the other number, rounded by placing a one in its least significant digit.

5) Division program extracting two digits  per major cycle [in tracks 20-24, cards 140-142]

Similar to the above and it can be tested in the same way. 576 minor cycles or 18.4 ms.

6) Integer Division [in tracks 25-29, cards 143-145]

Divides the single length number in TS15 by the number in TS26, with the quotient in DS14 even and the remainder in DS14 odd. Uses ordinary restoring method, taking 32 steps, measured by shifting a bit in TS16. This was written to be used in the ‘smallest factor’ program. It takes 1056 minor cycles (33 major cycles) or 33.792 milliseconds.

7) Smallest Factor [in tracks 30-34, cards 146-148]

Given  a number in the input register, tries division by 3, 5, 7,... until the quotient is smaller than the test divisor. If the remainder is zero, signals a prime by the buzzer. If not, outputs the smallest factor of the given number and returns to the start in either case. When testing 999983 with the 499 numbers up to 999 it takes 542912 minor cycles or 17.37 seconds. This is 499 times 34 major cycles. The simulation takes about 8 seconds. If 2^31 - 1 = 2 147 483 647 is input, the proof that it is prime takes 25208960 minor cycles or 807 seconds on the Pilot Model and 369 seconds in the simulation. This is 23170 divisions, each of 34 major cycles, 2 being ‘overhead’.

The largest prime in range is 2^32 - 5 = 4294967291 which takes 35650496 m.c. or 19 minutes. In simulation it takes 8 minutes 45 seconds. Another test is 4293001441 which is the square of the prime 65521 and also takes 8 minutes 45 seconds.

8) Integer division, double speed [in tracks 35-39, cards 149-151]

A version of the program (7) with two steps per major cycle. It takes 544 minor cycles, which is 17 major cycles or 17.408 milliseconds.

9) Smallest factor with double speed division [in tracks 40-44, cards 152-154]

With the prime 999983 it takes 287424 minor cycles or 9.197 seconds. The simulation takes about 6.5 seconds. With the prime 2^32 - 5 it takes 18873792 minor cycles or 604 seconds. The simulation takes about 420 seconds.

10) Conversion of program to input cards [in tracks 45-49, cards 155-160]

It is convenient in this simulation to write and test a program in the delay lines and transfer it to the drum for storage. To load the program by initial input, it has to be split between three cards with the initial input program at its head. I have written a program which will convert one delay line full of program into three card images. It occupies DL1 and DL2. In the drum, DL1 is in tracks 155, 157 and 159, while DL2 is in tracks 156 and 158. The initial input occupies six cards, numbers 155 to 160.

Having loaded the program, the DL contents to be processed should be loaded into DL3. Enter the program at DL1, word 0 and immediately the buzzer will signal completion. The results are in DL4, DL5 and DL6 and can be copied manually into the desired card numbers. However they were written for the simplest case which was loading DL1 and entering the program at DL1, word 0. Also the last card of the triple has zeros in the 13th row. For other destinations and for triples which are not the last in the pack it is necessary to modify the card images before storing them.

Using this program, all the programs described in this note have been converted and their initial card is noted with their description.

11) Copying a set of cards [in tracks 50-54, cards 161-163]

The cards to be copied are those beginning with the number entered at ‘Read Card’ and they are copied to cards beginning at ‘Punch Card’. The number of cards to be read must be entered in the input register. Note that only the first 12 words in a card image can be copied. To alter the others, a manual drum write from a DL can be used.

The program uses the special form of punched output enabling it to punch cards in the region 128-255, where programs are kept. If that region is chosen the ‘buzzer’ will operate and must be cleared to continue.

The number of cards copied must be in the range 1 to 9, a safety feature. When one set has been copied, clicking the ‘Run’ button copies the next set in succession.

12) Finding all the factors of a number [in tracks 55-59, cards 164-166]

A number less than 232 is entered into the input register, then the program run. It finds all the factors, beginning at the smallest, and loads them into DL10, the smallest factor furthest down the list. A quirk in the program gives the last factor at the top of the list as 1 if the largest factor occurs more than once. Since the program has only one spare word in the delay line this cannot be corrected in a one DL program.

A good example is 2230 92870 = 23 x 19 x 17 x 13 x 11 x 7 x 5 x 3 x 2

The well known case 999997 = 1321 x 757 takes 7 secs. (13 secs. in the real machine.)

13) List cubes of numbers to 1625 in DL 10 [in tracks 60-64, cards 167-169]

This is a variant of program 2. It calculates squares and cubes of the numbers from 1 to 1625 and places them in DL 10, using the rotation so that the last result, 16253 is in word 0 while (1625-31)3 is in word 31. It takes about 4.5 seconds or 4.99 seconds in the real machine.

14) List 6th powers of numbers to 32 in DL 10 [in tracks 65-69, cards 170-172]

Another way to calculate successive powers is to sum them from constant differences, in this case a constant sixth difference of 720. Takes 133 ms. In the real machine but could probably have been reduced to 98 ms. by better optimisation.

15) Table of sin(x) to p/2 in steps of p/60 [in tracks 70-74, cards 173-178]

Using the power series up to x15, this program calculates 31 values of sin(x) and stores these as the integers 231sin(x) in drum track 125, with sin(0) in word 31 at the bottom and cos(p/2) in word 1 near the top. The results can be seen in DL9. These are calculated as binary fractions with the binary point one place from the left. They are best displayed in hexadecimal as values of ½ sin(x) with the hexadecimal point on the left.

16) Given x, calculates sin(x) [in tracks 75-79, cards 179-184]

This is the sine routine used in the previous program. It takes ½ x from the input register and outputs ½ sin(x). Both are binary fractions, with the binary point to the left of P32.

17) Table of cos(x) to p/2 in steps of p/60 [in tracks 80-84, cards 185-190]

Using the power series up to x14, calculate 31 values of cos(x) and stores these as the integers 231cos(x) in drum track 126, with cos(0) in word 31 and cos(p) in word 1. See program 15 above.

18) Table of sin2(x) + cos2(x) as a check [in tracks 85-89, cards 191-193]

Using the values of sin(x) and cos(x) stored in tracks 125 and 126, this program sums their squares and places the results in track 127. This checks the sin and cos routines. The values obtained, regarded as integers, are 230 times the actual results and differ from 230 by from +1 to -2, with a tendency to be low (average -0.90). Another check is to compare 231sin(x) with 231cos(p-x). When they are not equal, the cos is low by 1 or 2 and in one instance 3. This is within expected values. The results are sensitive to the value of ¼ x2 which successively multiplies the terms. This is rounded to a 32 bit fraction and if it is too large it increases ½ sin(x) a little and decreases ½ cos(x) rather more, which could account for the bias, except that the end results, for x = p, are exactly 1 and 0 respectively.

Deficiencies of the simulation

1) The multiplier

Used in the correct way, the multiplier of the simulation gives the correct result but the time it occupies is ignored. A strict simulation would allow the programmer to interfere with its operation by accessing DS14, though this was very rarely used. The ability to run programs in parallel with the multiplier makes it quite difficult to include the multiplier’s effect on total time. It could simply occupy 2 major cycles as a rough estimate, but this would misrepresent the parallelism. In the simulation, a check is made that TCB is off and DS14 even is cleared, though the real machine did not do this, it would simply make errors.

2) The drum

No attempt is made to account for the delay imposed for head shifting and the 9 ms taken by transfers. This could be done if the time allowed for head shifts was known as well as the precise nature of the interlock (what things the interlock prevents).

(The simulation gives the elapsed time of the real machine accurately from the entry into a program to its completion if the multiplier, drum or cards are not used within the program.)

3) Strange instructions

The simulation has been tested with instructions such as are commonly found in programs. The example of TS20 when TCA=1 showed how easy it was to make a mistake which affected only a few kinds of instruction (in that case 20-10). It is possible that some unusual instructions exist which are not properly covered. No attempt was made to deal with ‘pathological’ use of instructions.

4) Card reading and punching

The operator’s work of selecting and assembling a pack of cards for the reader cannot be simulated. In the simulation, the card images for input must be assembled in the files devoted to cards (numbers 128 to 383 in our scheme) and the number of the first card given in a text box. Punching produces card images with numbers in the range 256 to 383. This may overwrite existing ‘punched’ cards. Manual operations to copy sets of 32 words between delay lines and cards make useful card manipulations possible, but it is very different from the real thing.

The T.I.L. source will work correctly only if it is tested after the stop instruction which reads or punches a card row. Outside the time of card operations it usually gives ones.

5) Realism

Because a programmed display of all the store is necessarily very slow, the useful facility of looking at memory during running is lost. Only the output register can be seen during a run. Initial input does give a glimpse of progress, however.

At present, Visual Basic makes it impossible to have a clean halt from a  running program.

The controls look nothing like the real machine. This could be remedied with extensive use of graphics, but not by me. Some Pilot model facilities such as stopping on partially specified instructions, are missing.

The simulation has manual features not present in the real machine, such as loading words into delay lines and registers and transfers between delay lines and drum tracks or card images. Words cannot be seen in ‘backwards binary’ form but are available in hexadecimal, decimal and instruction format. In practice, program preparation, loading and testing and changes during testing are very easy with this simulation.

References

1) ‘An assessment of the system of optimum coding used on the pilot automatic computing engine at the National Physical Laboratory.’ J.H.Wilkinson, Proc.Roy.Soc. Volume A 248, pp.253-281, 1955.

2) ‘Programming the Pilot ACE: Early programming activity at the National Physical Laboratory.’ Martin Campbell-Kelly, Annals of the History of Computing Volume 3, No.2, pp.133-162, 1981.

LIST OF SOURCES AND DESTINATIONS

MemH and MemL locations are shown

0	Reader			Instruction		1 and 0 respectively

1	DL1			DL1			32-63

2	DL2			DL2			64-95

------

10	DL10			DL10			320-351

11	DL11			DL11			352-383

12	DS12			DS12			384-385

13	DS14/2		DS14 add		-

14	DS14			DS14			386-387

15	TS15			TS15			388

16	TS16			TS16			389

17	~TS26			TS16 add		-

18	TS26/2			TS16 subtract		-

19	TS26*2		MULT			-

20	TS20			TS20			390	(also extension of DL10)

21	TS26&TS27		set TCA		-

22	TS26 Xor TS27	DRUM		-

23	P17			set TCB		-

24	P32			Disc on sign		-

25	P1			Disc on zero		-

26	TC26			TC26			391

27	TC27			TC27			392

28	zero			Output Stat.		28

29	ones			Buzzer			-

30	T.I.L.			Stim. Punch		-

31	spare			Stim. Reader		-

The instruction layout is

N	S	D	C	W	T	G

(1)	3	5	5	2	5  (3)	5  (2)	1   total 32 bits

If C = 0, the transfer runs from M+W+2 to M+T*+2, a total of T*-W+1 minor cycles

If C = 1, there is just one transfer at M+W+2

If C = 3, there are two transfers starting at M+W+2

where M is the current minor cycle and if T<W, T* means T+32

then the next instruction comes at the time M+T*+2

note that if T = W-1, the full transfer takes 32 minor cycles

and if T = W it takes 1 minor cycle, but with C = 3 it takes 2 minor cycles and the next instruction is at M+T+34.