HOW TO DEMONSTRATE THE ACE PILOT MODEL SIMULATION

D.W.Davies, November 1999

The files you need to run the simulation program

These files can be supplied on a diskette:

C:\vb\Acepm3.exe		the simulation program

C:\vb\Vbrun100.dll		the necessary part of the Visual Basic software

C:\vb\Acedrum		drum contents holding ACE Pilot model programs

C:\vb\Acecards		contents of punched cards for reading in programs

C:\vb\Acepunch		punched cards which can be ‘punched’ by programs

There are also files containing the Visual Basic programs in Word or Text format and a Word document which describes the simulation. These will be on the diskette.

Some preliminary remarks before a demonstration

The Pilot model was a ‘Reduced Instruction Set’ or RISC machine and required some skill to program it efficiently. Nothing was done to make the machine language easy. These things made the simulation difficult and some departures from realism were inevitable.

A single Visual Basic form gives all the information about the state of the machine and controls to manipulate it. These are roughly like the indications and controls of the real machine. There are many ways in which the machine can be ‘misused’ and a few strange ways to use it that could be worth exploiting (such as reading from the multiplier before it had stopped). But in general these uses had effects which were never documented so I have not struggled to reproduce them accurately. Because much of its complexity was aimed at clever timing, the time elapsed during most operations has been shown correctly but there are exceptions to this, notably the multiplier, drum, reader and punch. For example, having been started, the reader or punch will respond correctly to stopped instructions (Go = 0) but it does not demand these in the times given by the two mechanisms - they can be at any time in the simulation but there must be exactly 12 of them for the 12 rows of the card.

I have at present eleven working programs but not all are suitable for a demonstration and some must be handled with care. I will describe the most useful ones.

Starting the demonstration

With the above-mentioned files in place, bring up C:\vb\Acepm3.exe with Windows Explorer and double click it, to execute. From this moment the simulation program will run and the form will be displayed. There may be complications with the resolution of the display and David Clayden is the expert on this.

To escape from the simulation click the ‘Exit’ button, but it is possible that this (and all the other buttons) will not respond. Whenever the program is engaged in servicing one of the demands contained in my Basic programs, it cannot see anything else. Normally this phase will pass when the program ends but sometimes a very long program or an infinite loop will be entered. To test if this has happened, try one of the innocuous buttons like ‘Up’ or ‘Down’. If nothing happens you are stuck. The only way out that I have found is <Control> + <Alt> + <Delete>. Then respond to the questions asked, by clicking ‘End Task’ and you should be back to the Windows Explorer window, from which you can start again.

Explaining the form

When it first appears the form is rather blank, because I couldn’t get the ‘Form Load’ program to enter my display routine. I suggest clicking ‘Dec’ or ‘Instr’ in the options on the third line, then perhaps ‘Hex’ again to get an easy display. The contents (all zeros) of the delay lines 1 and 2 will be displayed. They are not in binary as in the real machine but in Hex notation - because there wasn’t room for two binary displays.

Other delay lines can be selected by clicking A or B above the displays, but all memory will contain zeros at this time.

To see a real display click the ‘Read’ button on the fourth line up. The program that ran in May 1950 will appear in DL1, in Hex notation. This has been read from track 0 of the drum, where it resides, in the drum file mentioned above. Now  we can see a feature not present in the real machine, by choosing the ‘Instr’ option on the third line and seeing instructions nicely partitioned and with the fields in decimal.

At this stage, point out the display of all the ‘short tanks’ as well as the input and output registers and the instruction register. Mention that any of these can be altered manually using the buttons provided, as will be seen later.

The ‘Successive Digits’ program

This was the first program that ran. It was designed to test out the loading and operation of instructions, the single length accumulator TS16, input and output of a  simple kind, both kinds of jump instruction and shifting in TS26.

The output was a single binary digit, running along the lamps which showed the output register. The rate of travel was adjusted by choosing an input number which was aded to the accumulator until it spilled into the sign digit, then making one shift of the output digit.

Having this simple program, it became the last resort when testing the machine after a failure. If it couldn’t do ‘Suc. Digs.’ It was in  a poor state. So Suc. Digs. Became an institution and it is very appropriate to show it today, feeble though it really is.

My version of Suc. Digs. Has been loaded from the drum into DL1. There was no record of the one we used on the real machine, but this one does the same thing - as far as I can remember.

The program should be drawn out on Slide 1. Describe how it works and note that 2 major cycles are used in the outer loop but only one when it goes back to the start.

To set the input value choose option ‘Hex’ and write 001 into the top text box. This is in binary 0000 0000 0001 with the lower bits all zeros, which is P21 in our notation. It will have to be added 2048 times to TS16 before a shift takes place, taking about about 2 seconds in the real machine. Next click the ‘Input’ button and 0010 0000 will appear in the input register.

Hex output in the output register looks very different from the shifting bit, but is easy to follow. I have made the program stop after the bit shifts off the end, so that the controls can be used, or the process started again with a one-shot. The last output will be 8000 0000 because this output happened before the last shift and test.

To start the program, after being sure to load the input register, click ‘Enter’ and the first instruction will appear in the instruction register, which is always in instruction format, of course. Then click ‘Run’ and the process starts. You will not see the process taking place in the TS display, unfortunately, because the display is so slow that it would be tedious.

The rate of change depends on the processor used. In my PC the whole shift from P1 to P32 takes about 8 seconds, which is 8 times faster than the real machine. The simulation spends time only on the instructions is obeys, so it is fast when the main loop has only two instructions, as in this simple program. The minor cycle counter on the fifth line shows that 2,099,168 minor cycles have elapsed, which make 65,599 major cycles, or 216 + 63. This would take 67 seconds in the real machine and it represents 32 times round the outer loop with 2048 additions to the accumulator in each loop, the outer loop adding 2 more major cycles per shift with a speedier return at the last time, no attempt being made for fast operation.

To see how the shifting rate depends on the input value, type 01 into the top text box, verify that you are in Hex format and click ‘Instr.’, then ‘Run’ again (the program has returned to its start). The shifting bit will race through at 16 times the former speed. The minor cycle counter does not clear but in fact only 133,088 new minor cycles elapse or 4159 major cycles which is 212 + 63, as expected - a count of 128 per shift instead of 2048.

Cubes and fourth powers

These two programs build up cubes and fourth powers from the binomial expansion of (n + 1)3 and (n + 1)4 respectively. Slide 2 shows the program for calculating cubes starting from n = 0 and building up to n = 1625, where the cube is the largest that fits into the 32 bit word. Each loop occupies 2 major cycles and the whole process would have taken 3.328 seconds in the real machine. In my PC it took about 2.5 seconds. This shows how the simulation loses its speed advantage when there are many instructions in the loop and in particular when some of these are multiple operations.

To read the program from the drum, write ‘5’ into the track number on the fourth line up and click ‘Read’. Track 5 holds this program. It can be seen in DL1 in the ‘Instr’ format but for running the program choose ‘Decimal’. Next click ‘Enter’ to load the first instruction and the ‘Run’. After a short pause, the display will show the last set of values in TS15 (n), TS20 (n2) and TS26 (n3).

Then click ‘1-shot’ three times to output the three numbers and the program will return to its start, when ‘Run’ can be clicked to run it again.

The program on track 60 of the drum does the same calculation but places the cubes in DL 10 by rotating it. At the end, cubes from 1594 to 1625 are visible, taking about 4.5 second or 4.99 in the real machine.

A similar program for fourth powers is in track 10 and can be loaded, examined and run in the same way. Due to the word size it calculates the four powers for numbers from 1 to 255 and this takes 16350 minor cycles or 0.52 seconds in the real machine. The main loop is still 2 major cycles but it holds more instructions of which more are multiple operations, hence a slower simulation would be expected in this case. It takes a fraction of a second so proper comparison is not possible. When the program stops and displays, the powers are in TSs 27, 26, 15 and 20 respectively. Four one-shots will output them and the program is ready to go again.

Division Programs

Jim Wilkinson’s paper in Proc. Roy. Soc. Describes a remarkable division program for the pilot model. It handles positive or negative numbers, using non-restoring division. A double length number is divided by single length number giving a rounded off result and two kinds of error indication for badly chosen sizes of the numbers. I have copied this in track 15 and find that it takes 35.4 milliseconds on the real machine, which is excellent. But Wilkinson also gives a version with two steps per major cycle and this is on track 20 and takes 18.4 milliseconds, even more remarkable. But these are not suitable for demonstration because the data have to be loaded first and nothing much seems to happen.

I wanted an integer division program giving a quotient and remainder, and this is on track 25, with a double speed version on track 25 which occupies 17.4 milliseconds on the real machine. Neither is suitable for demonstration.

Smallest Factor demonstrated in 1950

On November 29th and 30th and December 1st 1950, the ACE Pilot Model was working well enough that a grand public demonstration was held. Slide 3 shows four of those who took part, but many others did the hard work of getting the machine into good shape. Day 1 for the popular press produced stories about the ‘electronic brain’. Day 2 was for the technical press and day 3 for invited guests including F.C.Williams, Tom Kilburn and Maurice Wilkes.

Finding the smallest factor

There were two programs. The ‘popular’ program took in a six figure decimal number and ‘gave its highest factor’. Since the output allowed only numbers up to 999, as can be seen in the slide, I think it was actually the smallest factor. A bottle of beer was offered to the press for finding any six figure prime. Whenever a multiple of three or seven was called out, Wilkinson responded ‘divisible by 3!’ or ‘divisible by 7’ before it was entered. This amazed the press more than the machine itself.

The ‘serious’ demonstration was something I wrote which traced rays through a lens. Since it didn’t handle skew rays it wasn’t really much use, but it sounded technical.

Using the double speed integer division program, I have tried to reproduce the popular demonstration in a program which is on drum track 40. It fills DL1 with only two slots to spare. I have made it to treat any odd number less than 232 . The program is shown in Slide 4, but with the simpler division program, because the full program takes up too much space. If it finds a prime it sounds the buzzer, otherwise it outputs the smallest factor. It divides the given number by successive odd numbers and stops to indicate a smallest divisor if the remainder is zero. This continues until the divisor is greater than or equal to the quotient, when it sounds the buzzer to announce a prime.

Load it by setting track 40 then ‘Read’ and ‘Enter’. Then go decimal and put the number to be processed in the top text box and click ‘Input’ before ‘Run’.

The largest prime that could have been used in the actual demonstration was 999,983 and this was well known to us at the time. In my program this took 9.2 seconds in the real machine or 6.5 on my PC for the simulation. I don’t know if the program used at the demonstration used double speed division but if not it would have take 17.4 seconds, according to my program. Another useful number for demonstration is 999,997 which is 757 x 1321.

The largest prime in the range of my program is 232 - 5 or 4,294,967,291 and this takes 10 minutes on the real machine or 7 minutes in simulation on my PC.

An interesting further development of this program can be loaded from drum track 55. This takes any number less than 232 and finds all its factors, which it places in the words of DL10. It also treats even numbers. Where there are small factors it may give 1 as the smallest factor. This program uses 31 instructions and just fits into DL1. Try 2230 92870. Using 231 = 21474 83648 will fill the delay line with 31 twos and a one.

Input from punched cards

One of the clever tricks used with the Pilot Model was the input from punched cards punched in binary. This was the standard method of introducing new programs and employed 3 cards for a DL full of program. There were 4 rows of card to spare out of the 36 available rows and these were at the start of the first card. The first one was wasted then three special rows did the whole job of loading the program. I have written a short note on how this worked but it is probably too difficult to explain at this kind of demonstration.

I have arranged for the simulation to carry out this method of loading programs. The triples of cards are in files that are organised like drum tracks.

For card input in general I arranged that the memory contents would be displayed at each row of the card. This is against the usual rule that display slows the operation of the simulation but in any case the card itself was slowing down the operation. It enables the process of input of card triples to be seen in operation, though this depend on the characteristics of the display and may not always be effective.

To avoid confusion with drum tracks, cards are numbered from 128 upwards. Those from 128 to 255 cannot be overwritten by a program and are used to hold card triples for input. (they can however be written manually.) The ‘Read Card’ text box can be set for the first card to be read and is at 128 by default. The three cards starting at number 128 hold the Suc. Digs. Program so to read these cards as initial input it is only necessary to click ‘Init. Cards’. If Instr. format has been chosen, look carefully at DL1 and the loading by successive card rows can be seen, while ‘Read card’ moves from 128 to 129 and 130. At the end of this process Suc. Digs. Is in DL1 and ‘Read Card’ is set at 131, where another triple begins. An indication on the card tells the machine whether to continue reading - in this case not to.

It is instructive to click ‘Init. Cards’ again and input the next program from cards 131-133 since these hold the program which calculates cubes etc. of numbers up to 1625. The triple has been set to enter this program and the first instruction is not a stopped instruction in this case, so the calculation goes ahead and after about 2.5 seconds produces the last answer of the series and stops.

By setting the Read Card number to 155, a two DL program is input from six cards. This is a program I use to make the card triples when I have already tested a program held on the drum. Thus I have changed the way in which programs are developed to suit the new facilities.

If you want to continue the demonstration, read from the triple beginning at card 152, which is the smallest factor program with double speed division. Then people can propose numbers to be tested. Even numbers can be used but the results are not always what you would expect, unless you think about how it works. For example 2n times a prime will be declared a prime.