Ace Pilot Model Initial Instructions

D.W.Davies  20th September 1999

Every computer in which the programs are held in a volatile memory has a problem to get started, since no program for data input is available. Some trick is used to get into place a very simple program. It can be as simple as possible because it can be used to bring in a more elaborate program. This was called ‘bootstrapping’. Early machines used various methods, some requiring special hardware. The ‘bootstrap’ method was especially clever in the ACE Pilot Model. It began its work with an empty instruction, consisting of all zeros. This particular instruction has the effect of transferring one word of data from source zero (the input) to destination zero (the instruction register) and, by means of the clever scheme, that was all that was necessary to load delay lines from punched cards. This note describes how the clever trick operated. We begin with a reminder of the instruction format.

The instruction contents are as follows:

Next instruction source N - specifies the delay line containing the next instruction

Source S

Destination D

Characteristic C - specifies a transfer of either many words or one word or two words

Wait number W - specifies the wait before a transfer begins

Timing number T - specifies the timing of the next instruction

Go digit G - if this is zero the machine halts until a ‘one-shot’ occurs

An instruction is written in the format  N  S - D  C  W  T  G

Timing is given in units of a minor cycle, which is the transfer time of one word. For the purpose of specifying words in a delay line, times are given modulo 32, but sometimes an actual elapsed time in minor cycles is given.

The timing of an instruction is described as follows:

It is set up at a time X determined by the previous instruction

Transfer always begins at the word X + W + 2

The next instruction always is taken from delay line N at word X + T + 2

If the characteristic C is zero, the transfer takes place continuously from X + W + 2

to X + T + 2, a total of T - W + 1 minor cycles. The number of words that are transferred is  T - W + 1. Note that if T < W then T - W + 33 words are transferred.

In the case C = 1 there is a single transfer at time X + W + 2.

If C = 3 there is a transfer of two words  beginning at X + W + 2. This case never happens in the initial instructions scheme.

Concerning the card reader only the following need be known:

A key on the control desk starts this bootstrap procedure by doing two things, it clears the instruction register and then starts the card reader running until all the cards in the hopper have been read. While the card reader is running, 32 column of the cards are being read. Whenever a row of the card is under the brushes, the binary number punched on this row is being presented at the input source, S = 0. At the right moment for reading a row, a one-shot is given, so any instruction waiting with Go digit zero will be obeyed at the next correct minor cycle timing.

The 32 words for a delay line are punched on the rows of three cards. The first 4 rows of this triple have special punching which is, in effect, the bootstrap program. With 12 rows per punched card there is just room for the 32 words to follow.

The magic instructions on the first card of a triple designed to load 32 words into delay line A are:

First row	0     0 - 0     0     0    0     0

Second row	A    0 - A    0  	  26  25    0

Third row	A    0 - A    1    30  31	    0

Fourth row	0     0 - A    1    30  31    0

The procedure resulting from these card rows will now be described. Note that all the instructions described have a Go digit of zero. Therefore they halt the machine until the next row of the card at which a one-shot occurs. We can therefore describe the procedure as each row causes an instruction to be obeyed.

Row 1

The instruction  0   0 - 0   0   0   0   0 has been set up at time 0. In the actual Pilot Model this was defined as minor cycle 0 by convention, but in the simulation it is actually the datum from which timing is measured and also the total elapsed time is measured.

This instruction is obeyed at time 2 and transfers the input to the instruction register, namely the instruction 0   0 - 0   0   0   0   0 which was on the first row of the card. This instruction is set up at time 2.

Row 2

The instruction 0   0 - 0   0   0   0   0 has been set up at time 2. This instruction is obeyed at time 4 and transfers the input to the instruction register, namely the instruction  A   0 - A   0   26  25   0 which was on the second row of the card. This instruction is set up at time 4.

Row 3

The instruction  A   0 - A   0   26  25   0  has been set up at time 4. This instruction is obeyed by transfers starting at time  0 and continuing to time 31, which send the input to delay line A. Thus the word on row 3 of the card is placed in all words of delay line A and this word reads  A   0 - A   1  30  31  0. The next instruction will come from delay line A at time 31.

Row 4

The instruction A   0 - A   1  30  31   0  from delay line A has been set up at time 31. It is a single transfer at time 31 from the input to delay line A. The result is to transfer the word  0   0 - A  1  30  31  0 from the fourth row of the card to delay line A in minor cycle 31. The next instruction will come from delay line A at minor cycle 0.

Row 5

The instruction  A   0 - A   1  30  31  0  has been set up at time 0. It is a single transfer at time 0 from the input to delay line A. The result is to transfer whatever is on row 5 of the card from the input to delay line A in minor cycle 0, overwriting the instruction that caused this operation. This is the beginning of the desired result, since row 5 held the instruction destined for line 0 of the program held in delay line A. The next instruction will come from delay line A at minor cycle 1.

Rows 6 to 35

The first of the three cards has rows numbered from 1 to 12. We continue the row numbering on card number two with rows 13 to 24. Card number three then has rows 25 to 36 holding the last 12 of the instructions destined for delay line A.

The instruction A   0 - A   1  30  31  0 was placed in each word of delay line A except the last one, which differs by having next instruction zero. These instructions, because W = 30, replace themselves by the words received from the input. They takes the next instruction from the next word in the delay line. Therefore the delay line is filled, word by word, with the data from the punched card. This continues until the last word is reached, where the next instruction is different.

Row 36

The instruction 0   0 - A   1  30  31  0 held in word 31 of the delay line  has the same effect as the others and replaces itself by the word from the input, which is row 12 of the third card. Its next instruction number is 0, which by convention means delay line 8. This word zero of delay line 8 is assumed to contain zero. Presumably this delay line would be the last to be loaded if, as would rarely happen, as many as 8 delay lines were to contain program.

The instruction 0   0 - 0   0   0   0   0 will be taken from minor cycle zero of delay line 8 and be set up at time 0.

The machine has now returned to the same state as it began with. Another delay line can now be loaded from the next three cards and so forth until the card pack is exhausted.

Row 36 of the last card in the pack

For the last three cards, the word in row 4 can be altered so that, when the last delay line has been filled, the machine will enter the program at the correct point.

Suppose that the entry point is word Y of delay line B. The word punched on line 4 of the last triple is   B   0 - A   1  30  Y-1  0. As before, the word from the card row is loaded into delay line A at word 31.

The next instruction is from delay line B at time  31 + (Y - 1) + 2, which is time Y, as required and the program has been entered correctly.