LIST OF SOURCES AND DESTINATIONS

MemH and MemL locations are shown

0	Reader			Instruction		1 and 0 respectively

1	DL1			DL1			32-63

2	DL2			DL2			64-95

------

10	DL10			DL10			320-351

11	DL11			DL11			352-383

12	DS12			DS12			384-385

13	DS14/2		DS14 add		-

14	DS14			DS14			386-387

15	TS15			TS15			388

16	TS16			TS16			389

17	~TS26			TS16 add		-

18	TS26/2			TS16 subtract		-

19	TS26*2		MULT			-

20	TS20			TS20			390	(also extension of DL10)

21	TS26&TS27		set TCA		-

22	TS26 Xor TS27	DRUM		-

23	P17			set TCB		-

24	P32			Disc on sign		-

25	P1			Disc on zero		-

26	TC26			TC26			391

27	TC27			TC27			392

28	zero			Output Stat.		28

29	ones			Buzzer			-

30	T.I.L.			Stim. Punch		-

31	spare			Stim. Reader		-

The instruction layout is

N	S	D	C	W	T	G

(1)	3	5	5	2	5  (3)	5  (2)	1   total 32 bits

If C = 0, the transfer runs from M+W+2 to M+T*+2, a total of T*-W+1 minor cycles

If C = 1, there is just one transfer at M+W+2

If C = 3, there are two transfers starting at M+W+2

where M is the current minor cycle and if T<W, T* means T+32

then the next instruction comes at the time M+T*+2

note that if T = W-1, the full transfer takes 32 minor cycles

and if T = W it takes 1 minor cycle, but with C = 3 it takes 2 minor cycles and the next instruction is at M+T+34, I think!